IoT PHASE 3

SMART WATER SYSTEM

PROJECT DESCRIPTION:

For the monitoring of water tanks used to supply water for drinking, irrigation, restrooms and cleaning, ultrasonics are placed at the top of the tanks. This sensor will send ultrasonic waves which would hit the surface of the water and get reflected  back. Depending on the time taken to reflect back from the water level the ultrasound calculates the water level.

Here, the ultrasound in interfaced with Raspberri Pi Pico microcontroller.  Four LEDs are connected as well as a relay for motor is connected.When  the sensor detects the distance travelled by the reflected wave is greater than 400 cm, green LED glows and motor relay is switched on, indicating that the water level in the tank is low. When the distance is between 200 cm and 400 cm,yellow LED glows indicating water level s the middle range. When the distance is less than 100 cm, red LED glows, indicating the high water level in tank.

SCREENSHOTS:

MICROPYTHON CODE:

from machine import Pin

import utime

trigger=Pin(3,Pin.OUT) #outpin pin

echo=Pin(2,Pin.IN) #input pin

lowLED=Pin(19,Pin.OUT)

mediumLED=Pin(20,Pin.OUT)

highLED=Pin(21,Pin.OUT)

motor=Pin(22,Pin.OUT)

lowLED.high()

mediumLED.high()

highLED.high()

motor.low()

def ultson(): #record ultrasonic (us) sensor data

trigger.low() # non-active sensor

utime.sleep_us(2) #wait

trigger.high() #active sensor

utime.sleep_us(5) #wait

trigger.low()

while echo.value()==0: #no echo pulse detected

signaloff=utime.ticks_us()

while echo.value()==1: #true echo pulse detected

signalon=utime.ticks_us()

timepassed=signalon-signaloff

distance=(timepassed*0.0343)/2  #speed of sound in cm/microseconds, half the travel time

print("The distance of sensor from water level is :",distance,"cm")

level = distance

if (level >= 400) :

lowLED.low()

mediumLED.high()

highLED.high()

motor.high()

elif (level > 200) and (level < 400):

lowLED.high()

mediumLED.low()

highLED.high()

elif (level < 100):

lowLED.high()

mediumLED.high()

highLED.low()

motor.low()

while True:

ultson()

utime.sleep(5)